# Full Maintenance Schedule

## Weekly

☐ Wipe X & Y rails clean. or blow off with air ensure

there is no foam around belts

☐ Inspect X/Y belts for wear.

☐ Check trolley alignment.

☐ Clean wire guides.

## Monthly

☐ Inspect axis ball bearings.

☐ Lubricate linear bearings (OF/OC).

☐ Test E-Stop cable .

☐ Inspect tension spring.

☐ Inspect belt tension both sides to be equal

☐ Check cooling fans.

## General / As Needed

☐ Clean proximity switches.

☐ Replace worn wires.

☐ Keep machine clean.

## Environment Requirements

• Temperature: 5–35°C.

• Humidity: 30–75% non-condensing.

• Proper grounding required.

## Weekly Sign-Off

Technician Name: ____________________________
Date: ____________________________
Supervisor Signature: ____________________________