# /Preventive Maintenance Schedule —  Overhead Crane

## Monthly PM (Qualified Technician)

• Hook inspection (deformation, cracks, latch condition)

• Wire rope/chain condition (broken strands, wear, kinks, corrosion)

• Sheave rotation and wear

• Hoist drum rope spooling

• Limit switch verification

• Pendant/radio control function

• Festoon/cable track condition

• Trolley/bridge travel check

Technician Name: ______________________________

Technician Signature: __________________________

Date: ____________________

## Quarterly PM (Qualified Technician)

• All monthly PM items

• Brake inspection and functional test

• Motor brush inspection

• Gearbox oil level check and leak inspection

• Wheel and bearing wear

• Couplings, shafts, and keys inspection

• Runway and bridge alignment visual check

• Overload protection device test

Technician Name: ______________________________

Technician Signature: __________________________

Date: ____________________

## Semiannual PM (Qualified Technician)

• All monthly + quarterly items

• Full structural component check

• Electrical panel cleaning and terminal inspection

• End stops and bumper inspection

• Runway bolts/clips inspection

• Bearing and sheave lubrication

• Drive system gear wear check

• Motor and gearbox performance evaluation

Technician Name: ______________________________

Technician Signature: __________________________

Date: ____________________

## Annual PM (Qualified Technician)

• All lower-level PM items

• Annual load test per OSHA/ASME requirements

• Structural inspection

• Hook and load-bearing NDT (if required)

• Complete electrical system inspection

• Review and update of maintenance schedule

Technician Name: ______________________________

Technician Signature: __________________________

Date: ____________________

# Daily Crane Operator Visual Inspection Checklist — 15-Ton Overhead Crane

Date: ____________________

Operator: ____________________

Crane ID/Location: ____________________

## Hook & Load Block

☐ Hook not bent or twisted

☐ No cracks or gouges

☐ Safety latch working

☐ Hook rotates freely

## Wire Rope / Chain

☐ No broken strands or links

☐ No kinks, birdcaging, or crushed areas

☐ Proper seating in drum/sheaves

☐ No corrosion or excessive wear

## Hoist Operation

☐ Smooth lifting and lowering

☐ No abnormal noise

☐ No load slipping or drifting

☐ Limits working during normal use

## Trolley & Bridge

☐ Smooth travel (no binding)

☐ No abnormal vibration or noise

☐ Bumpers in place

## Controls

☐ Pendant/remote functions correctly

☐ Emergency stop working

☐ No visible wire damage

## Electrical / Mechanical

☐ No visible leaks

☐ Festoon/cable track moves properly

☐ No burning smell or hot spots

## Environment / Safety

☐ Area below crane is clear

☐ Warning labels visible

☐ Load within rated capacity

## Issues Found (Operator Notes)

## Technician Resolution Sign-Off

Technician Name: ______________________________

Corrective Action Taken:

Technician Signature: __________________________

Date: ____________________